LED Blink:

void setup() {

pinMode(A0, OUTPUT);

digitalWrite(A0, HIGH);

}

void loop() {

}

void setup() {

pinMode(A0, OUTPUT);  // A0 output

}

void loop() {

digitalWrite(A0, HIGH);  // A0 pin ON

delay(1000);              // 1 second wait

digitalWrite(A0, LOW);   // A0 pin OFF

delay(1000);              // 1 second wait

}

Push Button Timer LED:

#define LED 5

#define BTN 8

void setup() {

pinMode(LED, OUTPUT);

pinMode(BTN, INPUT_PULLUP);

}

void loop() {

if(digitalRead(BTN) == LOW) {

digitalWrite(LED, HIGH);

delay(1000);

digitalWrite(LED, LOW);

}

}

Servo Motor:

#include <Servo.h>

Servo x;

int servoPin = 3;

void setup() {

x.attach(servoPin);

Serial.begin(9600);

// Test positions

x.write(150);

delay(1000);

}

void loop() {

}

Interfacing I2C OLED with Arduino Nano:

#include <Wire.h>

#include <Adafruit_SSD1306.h>

Adafruit_SSD1306 oled(128, 64, &Wire, -1);

int num1 = 0;

int num2 = 0;

void setup() {

Wire.begin();

oled.begin(SSD1306_SWITCHCAPVCC, 0x3C);

oled.clearDisplay();

oled.setTextSize(2);

oled.setTextColor(SSD1306_WHITE);

oled.display();

Serial.begin(9600);

Serial.println("Enter two numbers:");

}

void loop() {

if (Serial.available()) {

// read numbers only when Enter is pressed

num1 = Serial.parseInt();

num2 = Serial.parseInt();

// Clear leftover characters in serial buffer

while (Serial.available()) Serial.read();

int sum = num1 + num2;

oled.clearDisplay();

oled.setCursor(0, 0); oled.print("Num1: "); oled.print(num1);

oled.setCursor(0, 20); oled.print("Num2: "); oled.print(num2);

oled.setCursor(0, 40); oled.print("Sum: ");  oled.print(sum);

oled.display();

Serial.print("Sum: "); Serial.println(sum);

Serial.println("Enter two numbers:");

}

}

#include <Wire.h>

#include <Adafruit_SSD1306.h>

Adafruit_SSD1306 d(128, 64, &Wire, -1);

void setup() {

Wire.begin();

d.begin(SSD1306_SWITCHCAPVCC, 0x3C);

d.clearDisplay();

d.setTextSize(2);

d.setTextColor(SSD1306_WHITE);

d.setCursor(20, 25);

d.print("Hi");

d.display();

}

void loop() {}

#define SSD1306_NO_SPLASH   // Disable Adafruit's default splash logo

#include <Wire.h>            // I2C communication library

#include <Adafruit_SSD1306.h> // OLED display library

// Create an OLED object with 128x64 resolution

Adafruit_SSD1306 d(128, 64, &Wire, -1);

void setup() {

Wire.begin();

// Start I2C communication (Nano/Uno → SDA=A4, SCL=A5)

d.begin(SSD1306_SWITCHCAPVCC, 0x3C);

// Initialize OLED (0x3C = I2C address)

d.clearDisplay();

// Clear the screen

d.setTextSize(2);

// Set text size

d.setTextColor(SSD1306_WHITE);

// Set text color to white

d.setCursor(10, 25);

// Set cursor position on the screen

d.print("Hello UAP");

// Display text on the screen

d.display();

// Actually update the screen with the text (very important)

}

void loop() {

// Empty loop – nothing will change continuously

}

#include <Wire.h>

#include <Adafruit_SSD1306.h>

Adafruit_SSD1306 oled(128, 64, &Wire, -1);

int num1, num2;

void setup() {

Wire.begin();

oled.begin(SSD1306_SWITCHCAPVCC, 0x3C);

oled.clearDisplay();

oled.setTextSize(2);

oled.setTextColor(SSD1306_WHITE);

Serial.begin(9600);

Serial.println("Enter two numbers:");

}

void loop() {

if (Serial.available() > 0) {

num1 = Serial.parseInt();  // first number

num2 = Serial.parseInt();  // second number

int sum = num1 + num2;

oled.clearDisplay();

oled.setCursor(0, 0); oled.print("Num1: "); oled.print(num1);

oled.setCursor(0, 20); oled.print("Num2: "); oled.print(num2);

oled.setCursor(0, 40); oled.print("Sum: ");  oled.print(sum);

oled.display();

Serial.print("Sum: "); Serial.println(sum);

Serial.println("Enter two numbers:");

}

}

PWM Control:

int a = 2;

int b = 4;

int pwm = 3;

void setup() {

pinMode(a, OUTPUT);

pinMode(b, OUTPUT);

pinMode(pwm, OUTPUT);

// Start motor clockwise

digitalWrite(a, LOW);

digitalWrite(b, HIGH);

analogWrite(pwm, 200); // Speed 0-255

}

void loop() {

}

int a = 2;    // IN1

int b = 4;    // IN2

int pwm = 3;  // Enable / PWM pin

void setup() {

pinMode(a, OUTPUT);

pinMode(b, OUTPUT);

pinMode(pwm, OUTPUT);

}

void loop() {

// Motor clockwise

digitalWrite(a, LOW);

digitalWrite(b, HIGH);

analogWrite(pwm, 200);  // Speed 0-255

delay(1000);             // 1 second

// Motor anti-clockwise

digitalWrite(a, HIGH);

digitalWrite(b, LOW);

analogWrite(pwm, 220);  // Speed 0-255

delay(1000);             // 1 second

}

Photoresistor:

void setup() {

Serial.begin(9600);

}

void loop() {

int analogValue = analogRead(A5);

float voltage = analogValue * (5.0 / 1023.0);

Serial.println(voltage);

delay(100);

}

void setup() {

Serial.begin(9600);

}

void loop() {

Serial.println(analogRead(A5));

delay(100);

}

void setup() {

Serial.begin(9600);

pinMode(5, OUTPUT);

}

void loop() {

int analogValue = analogRead(A5);

float voltage = analogValue * (5.0 / 1023.0);

Serial.print(voltage);

Serial.println(" V");

if(voltage > 3.0) {

digitalWrite(5, HIGH);

Serial.println("Light Ditect.");

} else {

digitalWrite(5, LOW);

}

delay(100);

}

Gas Sensor:

int GAS_SENSOR = A0;

void setup() {

Serial.begin(9600);

delay(3000);

}

void loop() {

int gasValue = analogRead(GAS_SENSOR);

float voltage = gasValue * (5.0 / 1023.0);

Serial.print("Analog Value = ");

Serial.print(gasValue);

Serial.print(" & Voltage = ");

Serial.print(voltage);

Serial.println("V");

delay(1000);

}

int GAS_SENSOR = A0;

int ALARM = 12;

void setup() {

Serial.begin(9600);

pinMode(ALARM, OUTPUT);

delay(3000);

}

void loop() {

int val = analogRead(GAS_SENSOR);

float volt = val * (5.0 / 1023.0);

Serial.print("V: ");

Serial.print(volt, 2);

if(volt > 4.5) {

digitalWrite(ALARM, HIGH);

Serial.println("RED ALERT!");

}

else if(volt > 4.0) {

digitalWrite(ALARM, HIGH);

delay(100);

digitalWrite(ALARM, LOW);

Serial.println("ALERT");

}

else if(volt > 3.0) {

digitalWrite(ALARM, LOW);

Serial.println("SAFE");

}

else {

digitalWrite(ALARM, LOW);

Serial.println("SAFE");

}

delay(1000);}

Obstacle Detection:

int LED=13;

int PIRPin=11;

int isObstacle=LOW;

void setup()

{

pinMode(LED, OUTPUT);

pinMode(PIRPin, INPUT);

Serial.begin(9600);

}

void loop()

{

isObstacle=digitalRead(PIRPin);

if (isObstacle==HIGH){

Serial.println("OBSTACLE!!,OBSTACLE!!");

digitalWrite(LED, HIGH);

}

else{

Serial.println("Clear");

digitalWrite(LED, LOW);

}

delay(1000);

}

Ultrasonic Distance Sensor:

void setup() {

Serial.begin(9600);

pinMode(13, OUTPUT);

}

void loop() {

//Send pulse to sensor

pinMode(11, OUTPUT);      // Set pin 11 as OUTPUT mode

digitalWrite(11, LOW);    // First set to LOW (0V)

delayMicroseconds(2);     // Wait 2 microseconds

digitalWrite(11, HIGH);   // Then set to HIGH (5V)

delayMicroseconds(10);    // Keep HIGH for 10 microseconds

digitalWrite(11, LOW);    // Set back to LOW

//Read Pulse

pinMode(11, INPUT);

long t = pulseIn(11, HIGH);

//Distance

int d = t * 0.034 / 2;

//Show Distance In Serial Monitor.

if(d > 0) {

Serial.print("Distance: ");

Serial.println(d);

}

//LED

digitalWrite(13, (d < 100) ? HIGH : LOW);

delay(500);

}